| SECTION 1: CHEMICAL PRODUCT AND COMPANY IDENTIFICATION |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product name |  | SPU 6-22 Use : De-ruster |  |  |  |  |  |  |  |
| Manufacturer’s Name |  | : UNASCO (M) SDN.BHD. |  |  |  |  |  |  |  |
| Address |  | : No.1, Jln PJU 1A/18, Tmn Perindustrian Jaya Subang, 46050 Petaling Jaya, Selangor. |  |  |  |  | Tel No : 03-78473468 |  |  |
| Emergency Tel. No. |  | : 03-78473468 |  |  |  |  |  |  |  |
| Contact point |  | : Technical Manager |  |  |  |  | Tel. No. : 03-78473468 |  |  |
| SECTION 2 : COMPOSITION/INFORMATION OF INGREDIENT |  |  |  |  |  |  |  |  |  |
| Chemical Name |  |  | CAS No. |  |  | Proportion |  |  | Exposure Limit |
| Phosphoric acid |  |  | 7664-38-2 |  |  | >20% |  |  | 1 mg/m3 |
| SECTION 3 : PHYSICAL AND CHEMICAL PROPERTIES |  |  |  |  |  |  |  |  |  |
| Appearance |  |  |  | Green liquid |  |  |  |  |  |
| Solubility in water |  |  |  | Complete |  | Melting Point |  | N/A oC |  |
| Boiling Point |  |  |  | N/A | o C | Flash Point |  | N/A oC |  |
| Auto-ignition temperature |  |  |  | N/A | o C | Flammable Limit |  | Non flammable |  |
| Specific Gravity |  |  |  | 1.320 |  | pH Value |  | <1 (Neat) |  |
| SECTION 4 : HAZARD IDENTIFICATION |  |  |  |  |  |  |  |  |  |
| Most important hazard : Cause burns. Risk of serious damage to eyes. |  |  |  |  |  |  |  |  |  |
| SECTION 5 : FIRST AID MEASURE |  |  |  |  |  |  |  |  |  |
| Ingestion | Do not induce vomiting. Give large quantities of water and seek medical attention immediately. |  |  |  |  |  |  |  |  |
| Eye contact | Immediately flush with running water for at least 15 minutes. Seek medical attention without delay. |  |  |  |  |  |  |  |  |
| Skin contact | Immediately remove contaminated clothing and shoes-launder before re-use. Wash skin thoroughly with water. Seek medical advice if redness and irritation occurs. |  |  |  |  |  |  |  |  |
| Inhalation | Remove to fresh air. Give artificial respiration if not breathing. Seek medical attention immediately. |  |  |  |  |  |  |  |  |

| SECTION 6 : FIRE FIGHTING MEASURE |  |  |  |
| --- | --- | --- | --- |
| Extinguishing Media |  | Select extinguishing media suitable for surrounding fire. |  |
| Fire fighting instruction |  | Wear self-contained breathing apparatus and protective clothing. Cool fire exposed container with water spray. |  |
| Special Hazard |  | Contact with most metals causes formation of flammable and explosive hydrogen gas. |  |
| SECTION 7 : ACCIDENTAL RELEASE MEASURE |  |  |  |
| Leak and / or Spill | Ventilated area of spill. Wear appropriate personal protective equipment. Contain and recover liquid when possible. Neutralize with alkaline material (soda ash, lime), then absorb with an inert material (dry sand, earth), and place in a chemical waste container for disposal. Wash spillage area with plenty of water. |  |  |
| SECTION 8 : HANDLING AND STORAGE |  |  |  |
| Handling | When diluting ALWAYS ADD ACID TO WATER, NEVER WATER TO ACID. Avoid contact with eyes and skin. Wash thoroughly after handling. |  |  |
| Storage | Store in a cool, dry, ventilated storage area. Keep out of direct sunlight and away from heat, and incompatible materials. |  |  |
| SECTION 9 : EXPOSURE CONTROL AND PERSONAL PROTECTION |  |  |  |
| a. Exposure Limit |  |  | ACGIH: TLV: phosphoric acid: 1 mg/m3 (TWA). |
| b. Ventilation |  |  | Use general or local exhaust ventilation to meet exposure limit requirement. Use only with adequate ventilation. |
| c. Personal protection Eye |  |  | Use chemical safety goggles and /or face shield where splashing is possible. Maintain eye wash fountain and quick-drench facilities in work area. |
| Skin |  |  | Use protecting clothing, gloves, boots to prevent body contact with the liquid. |
| Respiratory protection |  |  | Use NIOSH approved respirator in absence of proper ventilation. |
| SECTION 10: STABILITY AND REACTIVITY |  |  |  |
| Condition to avoid |  |  | Incompatibles. |
| Incompatibles |  |  | Strong base, most metals, cyanides etc. |
| Decomposition products |  |  | Phosphorus oxides may form when heated to decomposition. |
| Hazardous Polymerization |  |  | Will not occur. |
| SECTION 11 : ECOLOGICAL INFORMATION |  |  |  |
| Mobility |  |  | Soluble in water. |
| Bioaccumulation |  |  | N/A |
| Biodegradability |  |  | N/A |
| Aquatic Toxicity |  |  | High concentration will injure aquatic life by the effect on pH. Do not discharge directly into river, pond or stream. |

| SECTION 12 : TOXICOLOGICAL INFORMATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Toxicity Data |  | Phosphoric acid Oral LD50 (rat) : 1530 mg/kg |  |  |
| Carcinogenicity |  | N/A |  |  |
| Effects of overexposure Route of entry; Effects. Inhalation; Can cause irritation to nose, throat and upper respiratory tract.. Skin contact; Corrosive. May cause redness, pain, and severe skin burns.. Eye contact; Corrosive. May cause redness, pain, blurred vision, eye burns, and permanent eye damage.. Ingestion; Corrosive. May cause sore throat, abdominal pain, nausea, and severe burns of the mouth, throat, and stomach.. Chronic Effects No information found. |
| Target Organs |  | N/A |  |  |
| Medical conditions generally aggravated by exposure |  |  |  | N/A |
| SECTION 13 : DISPOSAL INFORMATION |  |  |  |  |
| Disposal of waste to be in accordance with the Environmental Quality ( Scheduled Wastes) Regulation 2005 and other guidelines issued by DOE and /or local authorities. |  |  |  |  |
| SECTION 14 : TRANSPORT INFORMATION |  |  |  |  |
| UN/NA Number |  | UN 3264 |  |  |
| T.D.G classification |  | Class 8 |  |  |
| Packing group |  | III |  |  |
| Special Shipping instruction |  | N/A |  |  |
| SECTION 15 : REGULATORY INFORMATION |  |  |  |  |
| a. Proposed classification : |  |  | Corrosive. |  |
| b. Risk phrase : | R34 |  | Cause burns. |  |
|  | R41 |  | Risk of serious damage to eyes. |  |
| c. Safety phrase : | S26 |  | In case of contact with eyes, rinse immediately with large amount of water and seek medical advice. |  |
| SECTION 16: OTHERS INFORMATION |  |  |  |  |
| The data given here is based on our current knowledge and experience. The purpose of this Safety Data sheet is to describe the product in terms of their health and safety requirements. Since the conditions of use are outside our UNASCO’s control, any recommendations or suggestions are made without guarantee and UNASCO disclaims any liability for loss or damage suffered from use of this information. Users are responsible for safe working. |  |  |  |  |