| SECTION 1: CHEMICAL PRODUCT AND COMPANY IDENTIFICATION |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product Name |  | SPU 6-93R Use : Zinc phosphating solution |  |  |  |  |  |  |  |
| Manufacturer’s Name |  | : UNASCO (M) SDN.BHD. |  |  |  |  |  |  |  |
| Address |  | : No.1, Jln PJU 1A/18, Tmn Perindustrian Jaya Subang, 46050 Petaling Jaya, Selangor. |  |  |  |  | Tel. No : 03-7847 3468 |  |  |
| Emergency Tel. No. |  | : 03-7847 3468 |  |  |  |  |  |  |  |
| Contact Point |  | : Technical Manager |  |  |  |  | Tel. No. : 03-7847 3468 |  |  |
| SECTION 2 : COMPOSITION/INFORMATION OF INGREDIENT |  |  |  |  |  |  |  |  |  |
| Chemical Name |  |  | CAS No. |  |  | Proportion |  |  | Exposure Limit |
| Phosphoric acid |  |  | 7664-38-2 |  |  | >10% |  |  | 1 mg/m3 |
| Nitric acid |  |  | 7697-37-2 |  |  | >5% |  |  | 2 ppm |
| SECTION 3 : PHYSICAL AND CHEMICAL PROPERTIES |  |  |  |  |  |  |  |  |  |
| Appearance |  |  |  | Green liquid |  |  |  |  |  |
| Solubility in water |  |  |  | Complete |  | Melting Point |  | N/A oC |  |
| Boiling Point |  |  |  | N/A | o C | Flash Point |  | N/A oC |  |
| Auto-ignition temperature |  |  |  | N/A | o C | Flammable Limit |  | Non-flammable |  |
| Specific Gravity |  |  |  | 1.516 |  | pH Value |  | 2.5±0.5 (1% w/v solution) |  |
| SECTION 4 : HAZARD IDENTIFICATION |  |  |  |  |  |  |  |  |  |
| Most important hazard : Contact with combustible material may cause fire. Cause severe burns. Risk of serious damage to eyes. |  |  |  |  |  |  |  |  |  |
| SECTION 5 : FIRST AID MEASURE |  |  |  |  |  |  |  |  |  |
| Ingestion | Do not induce vomiting. Give two glasses of water and seek medical attention immediately. |  |  |  |  |  |  |  |  |
| Eye contact | Immediately flush with running water for at least 15 minutes. Seek medical attention without delay. |  |  |  |  |  |  |  |  |
| Skin contact | Immediately remove contaminated clothing and shoes-launder before re-use. Wash skin thoroughly with water. Seek medical advice if redness and irritation occurs. |  |  |  |  |  |  |  |  |
| Inhalation | Remove to fresh air. Give artificial respiration if not breathing. Obtain medical attention. |  |  |  |  |  |  |  |  |

| SECTION 6 : FIRE FIGHTING MEASURE |  |  |
| --- | --- | --- |
| Extinguishing Media | Select extinguishing media suitable for surrounding fire. |  |
| Fire fighting instruction | Wear self-contained breathing apparatus and protective clothing. Cool fire exposed container with water spray. |  |
| Special Hazard | Oxidizer. Contact with combustible material, flammable material, or powder metals can cause fire or explosion. Reacts with most metals to release hydrogen gas. |  |
| SECTION 7 : ACCIDENTAL RELEASE MEASURE |  |  |
| Leak and / or Spill | Ventilate area of leak. Wear appropriate personal protective clothing. Contain spillage by damming or construction of barriers. Neutralize with alkali (e.g. Soda ash), then absorb with earth or non-organic absorbent material and place into dry welllabeled container for disposal. Wash spillage area with plenty of water. |  |
| SECTION 8 : HANDLING AND STORAGE |  |  |
| Handling | Open slowly to release potential pressure before removing cap. When diluting, the acid should always be added slowly to water and in small amounts. Avoid contact with skin and eyes. Wash thoroughly after handling. |  |
| Storage | Store in a cool, ventilated storage area. Keep out of direct sunlight and away from heat, water and incompatible materials. |  |
| SECTION 9 : EXPOSURE CONTROL AND PERSONAL PROTECTION |  |  |
| a. Exposure Limit |  | ACGIH TLV : Nitric acid : 2.0 ppm (TWA) Phosphoric acid: 1 mg/m3 (TWA) |
| b. Ventilation |  | A system of local and/or general exhaust is recommended to keep employee exposures below the Airborne Exposure Limits. |
| c. Personal protection Eye |  | Use chemical safety goggles and/or a full face shield where splashing is possible. Maintain eye wash fountain and quick-drench facilities in work area. |
| Skin |  | Wear impervious protective clothing, including boots, gloves, apron or coveralls, as appropriate, to prevent skin contact. |
| Respiratory protection |  | If the exposure limit is exceeded, wear a NIOSH approved respirator. |
| SECTION 10: STABILITY AND REACTIVITY |  |  |
| Condition to avoid |  | Stable under ordinary conditions of use and storage. Avoid light and heat. |
| Incompatibles |  | Strong bases, metallic powder, carbides and combustible organics. |
| Decomposition products |  | N/A |
| Hazardous Polymerization |  | Will not occur. |
| SECTION 11 : ECOLOGICAL INFORMATION |  |  |
| Mobility |  | Soluble in water. |
| Bioaccumulation |  | N/A |
| Biodegradability |  | N/A |
| Aquatic Toxicity |  | This material is expected to be toxic to aquatic life. |

| SECTION 12 : TOXICOLOGICAL INFORMATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Toxicity Data |  | Nitric acid LC50 inhalation (rat) :244 ppm(NO2)/30M Phosphoric acid Oral LD50 (rat) : 1530 mg/kg |  |  |
| Carcinogenicity |  | N/A |  |  |
| Effects of overexposure Route of entry; Effects. Inhalation; Corrosive.. Skin contact; Corrosive. Can cause redness, pain and severe skin burns.. Eye contact; Corrosive. Contact may cause severe burns and permanent eye damage.. Ingestion; Corrosive. Swallowing can cause immediate pain and burns of mouth, throat, esophagus and gastrointestinal tract.. Chronic Effects N/A |
| Target Organs |  | N/A |  |  |
| Medical conditions generally aggravated by exposure |  |  |  | N/A |
| SECTION 13 : DISPOSAL INFORMATION |  |  |  |  |
| Disposal of waste to be in accordance with the Environmental Quality ( Scheduled Wastes) Regulations 2005 and other guidelines issued by DOE and /or local authorities. |  |  |  |  |
| SECTION 14 : TRANSPORT INFORMATION |  |  |  |  |
| UN/NA Number |  | UN3264 |  |  |
| T.D.G classification |  | Class 8 – Corrosive (Liquid) |  |  |
| Packing group |  | II |  |  |
| Special Shipping instruction |  | N/A |  |  |
| SECTION 15 : REGULATORY INFORMATION |  |  |  |  |
| a. Proposed classification : |  |  | Corrosive |  |
| b. Risk phrase : | R8 |  | Contact with combustible material may cause fire. |  |
|  | R41 |  | Risk of serious damage to eyes. |  |
|  | R35 |  | Cause severe burns. |  |
| c. Safety phrase : | S23 |  | Do not breathe fume. |  |
|  | S26 |  | In case of contact with eyes, rinse immediately with large amount of water and seek medical advice. |  |
|  | S36 |  | Wear suitable protective clothing. |  |
| SECTION 16: OTHERS INFORMATION |  |  |  |  |
| The data given here is based on our current knowledge and experience. The purpose of this Safety Data sheet is to describe the product in terms of their health and safety requirements. Since the conditions of use are outside our UNASCO’s control, any recommendations or suggestions are made without guarantee and UNASCO disclaims any liability for loss or damage suffered from use of this information. User is responsible for safe working. |  |  |  |  |