| SECTION 1: CHEMICAL PRODUCT AND COMPANY IDENTIFICATION |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product Name |  | SPU 9-31 Use : Accelerator for zinc phosphating |  |  |  |  |  |  |
| Manufacturer’s Name |  | : UNASCO (M) SDN.BHD. |  |  |  |  |  |  |
| Address |  | : No.1, Jln PJU 1A/18, Tmn Perindustrian Jaya Subang, 46050 Petaling Jaya, Selangor. |  |  |  | Tel No : 03-7847 3468 |  |  |
| Emergency Tel. No. |  | : 03-7847 3468 |  |  |  |  |  |  |
| Contact Point |  | : Technical Manager |  |  |  | Tel. No. : 03-7847 3468 |  |  |
| SECTION 2 : COMPOSITION/INFORMATION OF INGREDIENT |  |  |  |  |  |  |  |  |
| Chemical Name |  |  | CAS No. |  | Proportion |  |  | Exposure Limit |
| Sodium nitrite |  |  | 7632-00-0 |  | >5% |  |  | Not established |
| SECTION 3 : PHYSICAL AND CHEMICAL PROPERTIES |  |  |  |  |  |  |  |  |
| Appearance |  |  |  | Clear, pale yellow liquid |  |  |  |  |
| Solubility in water |  |  |  | Completely | Melting point |  | N/A oC |  |
| Boiling Point |  |  |  | N/A oC | Flash Point |  | N/A oC |  |
| Auto-ignition temperature |  |  |  | N/A oC | Flammable limit |  | Non-flammable |  |
| Specific Gravity |  |  |  | 1.201 | pH Value (Neat) |  | 9 |  |
| SECTION 4 : HAZARD IDENTIFICATION |  |  |  |  |  |  |  |  |
| Most important hazard : Contact with combustible material may cause fire. Toxic by inhalation, in contact with skin and if swallowed. |  |  |  |  |  |  |  |  |
| SECTION 5 : FIRST AID MEASURE |  |  |  |  |  |  |  |  |
| Ingestion | Do not induce vomiting. Give large quantities of water and seek medical attention immediately. |  |  |  |  |  |  |  |
| Eye contact | Immediately flush with running water for at least 15 minutes. Seek medical attention without delay. |  |  |  |  |  |  |  |
| Skin contact | Immediately remove contaminated clothing and shoes-launder before re-use. Wash skin thoroughly with water. Seek medical advice if redness and irritation occurs. |  |  |  |  |  |  |  |
| Inhalation | Remove to fresh air. Give artificial respiration if not breathing. If breathing difficult, oxygen should be given by qualified personnel. Seek medical attention immediately. |  |  |  |  |  |  |  |

| SECTION 6 : FIRE FIGHTING MEASURE |  |  |  |
| --- | --- | --- | --- |
| Extinguishing Media |  | Water or water spray in early stages of fire. Foam may be used. |  |
| Fire fighting instruction |  | Wear self-contained breathing apparatus and protective clothing. |  |
| Special Hazard |  | Contact with oxidizable substances may cause violent combustion. |  |
| SECTION 7 : ACCIDENTAL RELEASE MEASURE |  |  |  |
| Leak and / or Spill | Ventilated area of spill. Wear appropriate personal protective equipment. Remove all sources of ignition. Large spill should be absorbed with inert material and place in a closed container for disposal. Wash spill area with plenty of water. |  |  |
| SECTION 8 : HANDLING AND STORAGE |  |  |  |
| Handling | Avoid contact with eyes and skin. Wash thoroughly after handling. |  |  |
| Storage | Keep in a tightly closed container. Store in a cool, dry, ventilated storage area. Isolate from any source of heat or ignition. |  |  |
| SECTION 9 : EXPOSURE CONTROL AND PERSONAL PROTECTION |  |  |  |
| a. Exposure Limit |  |  | Not established. |
| b. Ventilation |  |  | Use only with adequate ventilation. |
| c. Personal protection Eye |  |  | Use chemical safety goggles and /or face shield where splashing is possible. Maintain eye wash fountain and quick-drench facilities in work area. |
| Skin |  |  | Use protecting clothing, gloves, boots to prevent body contact with the liquid. |
| Respiratory protection |  |  | For emergency or instances where the exposure levels are not known, use a full-face positive-pressure, air-supplied respirator. |
| SECTION 10: STABILITY AND REACTIVITY |  |  |  |
| Condition to avoid |  |  | Heat, flame, ignition sources, incompatibles. |
| Incompatibles |  |  | Reacts vigorously with reducing materials and is incompatible with many substances including ammonium salts, cellulose, cyanides, urea, wood and etc. |
| Decomposition products |  |  | May form oxides of nitrogen. |
| Hazardous Polymerization |  |  | Will not occur. |
| SECTION 11 : ECOLOGICAL INFORMATION |  |  |  |
| Mobility |  |  | Soluble in water. |
| Bioaccumulation |  |  | N/A |
| Biodegradability |  |  | N/A |
| Aquatic Toxicity |  |  | No information found. |

| SECTION 12 : TOXICOLOGICAL INFORMATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Toxicity Data |  | Sodium nitrite Oral LD50 (rat) : 180 mg/kg |  |  |
| Carcinogenicity |  | N/A |  |  |
| Effects of overexposure Route of entry; Effects. Inhalation; Toxic. Causes irritation respiratory tract.. Skin contact; Causes irritation, redness and pain. May be absorbed through the skin causing systemic poisoning; symptoms may parallel ingestion.. Eye contact; Causes irritation, redness and pain.. Ingestion; Toxic if swallowed.. Chronic Effects No information found. |
| Target Organs |  | N/A |  |  |
| Medical conditions generally aggravated by exposure |  |  |  | N/A |
| SECTION 13 : DISPOSAL INFORMATION |  |  |  |  |
| Disposal of waste to be in accordance with the Environmental Quality ( Scheduled Wastes) Regulation 2005 and other guidelines issued by DOE and /or local authorities. |  |  |  |  |
| SECTION 14 : TRANSPORT INFORMATION |  |  |  |  |
| UN/NA Number |  | UN 3387 |  |  |
| T.D.G classification |  | 6.1 – Toxic (Liquid) , 5.1- Oxidizing |  |  |
| Packing group |  | I |  |  |
| Special Shipping instruction |  | N/A |  |  |
| SECTION 15 : REGULATORY INFORMATION |  |  |  |  |
| a. Proposed classification : |  |  | Toxic, Oxidizing |  |
| b. Risk phrase : | R8 |  | Contact with combustible material may cause fire. |  |
|  | R23/24/25 |  | Toxic by inhalation, in contact with skin and if swallowed. |  |
| c. Safety phrase : | S44 |  | If you feel unwell, seek medical advice (show the label if possible). |  |
| SECTION 16: OTHERS INFORMATION |  |  |  |  |
| The data given here is based on our current knowledge and experience. The purpose of this Safety Data sheet is to describe the product in terms of their health and safety requirements. Since the conditions of use are outside our UNASCO’s control, any recommendations or suggestions are made without guarantee and UNASCO disclaims any liability for loss or damage suffered from use of this information. Users are responsible for safe working. |  |  |  |  |